EV RECOMMENDED TOOLS AND SAFETY EQUIPMENT

Safety Equipment

| Image | Item | Link *Preferred | Cost |
| --- | --- | --- | --- |
|  | Class 0, Insulated rubber gloves kit -Rubber gloves, and protective leather gloves in a storage bag -Rubber gloves lifespan 6 months -Different sizes: Size 8-10 covers most people | Grainger Amazon AESWave* How to size gloves | $150-300 Per Set |
|  | Cotton under gloves (optional) | Grainger* Amazon | $1-6 Per Pair |
|  | Glove Dust (optional) | Grainger* | $15 |
|  | Rubber Glove Tester -Used to field test gloves before use | Grainger Amazon* | $350-600 |
|  | HV Rescue Hook (Hot stick) -Life span: 2 years | Grainger Amazon AESWave* | $500-750 |
|  | Arc Flash Clothing Kit -Some manufacturers require -Different sizes available -Category 2 (Might need higher depending on the job being performed) *See the manufacturer service manual for the recommended level of PPE | Grainger* Amazon | $700-1000 |
|  | Arc Flash Face Shield -Some manufacturers require | Grainger Amazon* | $72-$470 |
|  | HV Signage -You’ll need multiple signs around the vehicle and/or shop | Grainger Amazon* | $12-25 each |
|  | HV Barrier -Alternative barriers are possible with tall construction cones with caution tape between them | Grainger* Alternative: Grainger 4x Cone Grainger Caution tape | $1300-1500 $35 each $65 |
|  | Large NFPA 701 Certified Fire Blanket (9M X 6M) | Vehicle Response Training (VRT) Use code: Educator250 For a $250 discount | $2,800 |
|  | Class D Fire Extinguisher – Combustible Metals -Designed to extinguish nickel metal hydride batteries NOT effective on lithium chemistry batteries | Grainger ULINE | $900-1,100 |
|  | Lithium Battery Encapsulator Fire Extinguisher -Designed to combat lithium battery fires. Only use if the fire is small and containable. Place fire blank over component/vehicle ASAP. | All Hands Public Safety | $300-900 |
|  | Lock Box -Lock vehicle key and MSD away when HV system is being serviced | Grainger Amazon* | $32-70 |
|  | Lock Out, Tag Out Kit | Grainger Amazon* | $33-182 |

Basic EV Tools and Equipment

| Image | Item | Link *Preferred | Cost |
| --- | --- | --- | --- |
|  | CATIII Multi Meter with insulation tester – Fluke 1587 | Grainger Amazon* AESWave | $750-1100 |
|  | Proving unit FlukePRV240 -Alternative 9-volt battery | Grainger Amazon* | $160-170 |
|  | Level 1 & 2 NACS(Tesla) Charger | Amazon* | $300 |
|  | Level 2 J1772 Charger -You’ll need an electrician to add a 240 Volt plug/hardwire charging unit | Amazon* NorthernTool | $550-600 |
|  | Insulated Tool Kit | Grainger Amazon* NorthernTool | $530-2010 |
|  | OEM Factory Scan Tool -Recommend OEM factory scan tool for the EV you plan on working on the most. Example: Toyota TIS TechStream. | Toyota-TechStream | $6,330 |

Advanced EV Tools and Equipment

| Image | Item | Link *Preferred | Cost |
| --- | --- | --- | --- |
|  | 4-Channel Pico EV Kit 4425A -This is a great kit that has most of the things to scope HV components safely | AESWave* | $5,650 |
|  | Additional Differential Probes (optional) -Ideally 3 total | Amazon AESWave* | $460 |
|  | Milliohm Meter -Used to measure electrical resistance in motor windings | Amazon- Hioki ASEWave*- Hioki AESWave-PICO Milliohm Meter Explained | $1,100-1,400 $860 |

Other things to think about…

| Image | Item | Link |
| --- | --- | --- |
|  | Faraday Bag Adding a designated location for the key fob helps prevent the vehicle from unintentionally powering on when the key is within range. | Amazon*- Two Bag Set for $15 |
|  | HD Hoist (Wider) or Special EV hoist (optional) -Wider hoist to accommodate lowering HV Batteries | FleetMaintenance- Hoist Evolving BendPak- EV hoist Concept Rotary Hoist- EV lift new arms |
|  | EV Battery Lift Table and Fixtures -2,400-4,500LBS rated depending on what vehicles you have -Some tables will need additional fixtures to drop some batteries | BendPak- EV battery lift tables Hyundai-EV battery lift tables Challenger- Lift tables |
|  | Insulated Workbenches and Storage Shelves -Wood or nonconductive material to work on and store HV Batteries | Global Industrial- Wood workbench Global Industrial- ESD Tabletop |
|  | High-Voltage Module Balancer (optional) -If you plan on doing high-voltage battery internal servicing of modules, you’ll need the manufactured recommended balancer, Example: Ford’s recommended balancer from Medtronic -*Not all manufacturers are servicing HV battery packs | FORD Rotunda Midtronics Site and Video |